BRIGHTWATER BUILDERS GROUP, INC.

Corporate Safety & Health Program Manual

Revision 7  |  Effective March 1, 2025  |  Owner: Director of Safety & Health

## Revision log

| Rev | Date | Summary |
| --- | --- | --- |
| 5 | 2022-01-10 | Added silica section (Table 1) |
| 6 | 2023-06-01 | Fall protection raised to 6 ft for all trades |
| 7 | 2025-03-01 | Added healthcare infection control and ILSM; ladder policy; stop work restart rule |

This manual applies to Brightwater employees and to every subcontractor of any tier working on a Brightwater project. Subcontract Exhibit D incorporates it by reference.

# Section 1. Policy and Responsibilities

1.1  Brightwater Builders is committed to providing a workplace free of recognized hazards. Safety is a condition of employment and of every subcontract.

1.2  The Director of Safety & Health owns this manual, reviews it at least once a year and issues revisions with a revision log.

1.3  Every subcontractor shall designate a competent on-site safety representative for each project, by name, before mobilization.

1.4  Where this manual, the contract documents and OSHA differ, the most stringent requirement applies.

# Section 2. Subcontractor Prequalification

2.1  Subcontractors shall be prequalified before a subcontract is issued. Prequalification is valid for 24 months unless the subcontractor's safety performance changes materially.

2.2  Prequalification requires the three most recent years of EMR, OSHA 300A summaries and hours worked, a certificate of insurance and a copy of the written safety program.

2.3  An EMR above 1.00, a three-year average TRIR above 3.0, or any willful or repeat OSHA citation in the last three years requires written approval by the VP of Operations, with conditions.

2.4  Minimum insurance: general liability $2,000,000 per occurrence / $4,000,000 aggregate; workers' compensation at statutory limits with employer's liability of $1,000,000; umbrella of $5,000,000 for high-risk trades. Brightwater and the owner are named additional insured.

2.5  Certificates of insurance shall be current at all times. A subcontractor whose certificate lapses shall not work on site until a current certificate is received.

## Table 2-1. Prequalification thresholds

| Measure | Threshold | Above threshold |
| --- | --- | --- |
| EMR (current year) | 1.00 or less | VP of Operations approval with conditions |
| TRIR (3-year average) | 3.0 or less | VP of Operations approval with conditions |
| OSHA citations (3 years) | No willful or repeat | VP of Operations approval with conditions |

# Section 3. Site-Specific Safety Plans

3.1  Each subcontractor shall submit a site-specific safety plan within 10 working days of award and before mobilization.

3.2  The plan shall include the subcontractor's scope, hazard analysis for each high-risk activity, competent persons, emergency contacts and a training matrix.

3.3  Brightwater's Project Safety Manager reviews and approves the plan. Work may not start until it is approved.

3.4  Activity hazard analyses (AHAs) shall be completed before each new phase of work and reviewed with the crew.

# Section 4. Orientation and Training

4.1  Every person working on a Brightwater site shall complete the Brightwater site orientation before starting work.

4.2  On healthcare projects, workers in or next to occupied areas shall also complete the owner's infection control and hospital awareness module.

4.3  Subcontractors keep their own training records and shall produce them on request within 24 hours.

# Section 5. Fall Protection

5.1  Fall protection is required at 6 feet or more above a lower level for all trades and all activities, including steel erection and leading-edge work.

5.2  Scaffolds shall have guardrails at 6 feet; the OSHA 10-foot scaffold trigger does not apply on Brightwater sites.

5.3  Personal fall arrest systems shall be inspected before each use and anchored to anchors rated for 5,000 lb or designed by a qualified person.

5.4  Floor and roof openings shall be covered, secured and marked "HOLE" or "COVER" as soon as they are created.

# Section 6. Ladders and Aerial Lifts

6.1  Ladders are a last resort. Use of step or extension ladders above 6 feet requires the superintendent's approval of a ladder use plan.

6.2  Aerial lift operators shall be trained and familiarized on the model used; occupants of boom lifts shall be tied off to the manufacturer's anchor.

# Section 7. Excavation and Trenching

7.1  Before any digging, a Sunshine 811 locate ticket shall be current and private utilities located by a private locator.

7.2  Excavations 5 feet or deeper shall have a protective system; excavations over 20 feet shall have a protective system designed by a registered professional engineer.

7.3  A competent person shall inspect excavations daily, after every rain event and after any change in conditions, and record the inspection.

7.4  Spoil and equipment shall be kept at least 2 feet from the edge of an excavation.

# Section 8. Hot Work and Fire Prevention

8.1  All welding, cutting, brazing and grinding outside designated shop areas requires a Brightwater hot work permit.

8.2  Combustibles within 35 feet shall be removed or protected, and a fire watch shall be maintained during the work and for 60 minutes after it ends.

8.3  Hot work in or next to an occupied healthcare building also requires an interim life safety measures (ILSM) assessment and the owner's approval before the permit is issued.

8.4  Fire protection systems shall not be impaired without an approved impairment notice; the owner shall be notified and a fire watch posted for the duration.

# Section 9. Hazardous Energy Control

9.1  Lockout/tagout shall follow the subcontractor's written energy control program, reviewed by Brightwater, with individual locks for each worker.

9.2  Zero energy shall be verified by try-out and testing before work begins.

9.3  Tie-ins to existing hospital systems shall be coordinated through the owner's utility shutdown request process with at least 14 calendar days' notice.

# Section 10. Electrical Safety

10.1  Energized electrical work is prohibited unless justified under NFPA 70E and authorized on an energized electrical work permit signed by the subcontractor, Brightwater's Project Safety Manager and the Project Manager.

10.2  Temporary power shall be GFCI protected, and cords inspected daily.

# Section 11. Cranes and Rigging

11.1  A critical lift plan is required for any lift above 75% of the crane's rated capacity, tandem lifts, personnel lifts, and any lift over an occupied building or occupied area.

11.2  Critical lift plans shall be submitted at least 48 hours before the lift and approved by Brightwater's Project Safety Manager.

11.3  Loads shall not be lifted over the occupied hospital unless the owner has approved a written evacuation or protection plan for the affected area.

11.4  Operators shall be certified; riggers and signal persons qualified, with proof available on site.

# Section 12. Confined Spaces

12.1  Permit-required confined spaces shall be entered only under a written entry permit with atmospheric testing and a rescue plan.

# Section 13. Health Hazards

13.1  Silica-generating tasks shall follow Table 1 of 29 CFR 1926.1153 or an exposure assessment.

13.2  Work that disturbs ceilings, walls or floors in an existing hospital area requires an asbestos and lead survey review before work starts.

13.3  Noise and vibration next to occupied patient areas shall stay within the limits set by the owner, and shall be monitored when the owner requests it.

# Section 14. Personal Protective Equipment

14.1  Minimum PPE: hard hat (Type II on steel and crane work), safety glasses, high-visibility garment (Class 2), gloves suited to the task and safety-toe boots.

# Section 15. Housekeeping and Site Logistics

15.1  Work areas shall be cleaned daily; walkways and exits kept clear at all times.

15.2  Hospital entrances, ambulance routes and fire lanes shall never be blocked, even temporarily.

# Section 16. Infection Control on Healthcare Projects

16.1  An infection control risk assessment (ICRA) and ICRA permit shall be in place before any work in or next to an occupied healthcare area. The permit class sets the controls.

16.2  Class III and IV work requires hard barriers sealed floor to deck, negative air pressure verified and recorded at least daily, and HEPA-filtered exhaust.

16.3  Barriers shall be inspected daily by the subcontractor and weekly by the owner's infection prevention staff; any breach stops the work until it is repaired.

# Section 17. Inspections and Audits

17.1  The superintendent performs a documented weekly site inspection.

17.2  The Project Safety Manager audits each subcontractor at least monthly.

17.3  On healthcare projects, a joint monthly walk with the owner covers the connection areas.

# Section 18. Corrective Actions

18.1  Deficiencies are classed as Imminent danger, Serious, Other than serious, or Housekeeping / minor.

18.2  Imminent danger shall be corrected immediately and the work stopped until it is; Serious within 24 hours; all others within 7 days.

18.3  Corrective actions not closed on time are escalated to the subcontractor's project manager, then its principal after 2 days, then Brightwater's VP of Operations after 5 days.

## Table 18-1. Correction times

| Class | Correct within |
| --- | --- |
| Imminent danger | Immediately - stop the work |
| Serious | 24 hours |
| Other than serious | 7 days |
| Housekeeping / minor | 7 days |

# Section 19. Incident Reporting and Investigation

19.1  All injuries, near misses and property damage shall be reported to the superintendent immediately and in writing within 1 hour.

19.2  Serious incidents are investigated to root cause within 72 hours with a written report.

19.3  Subcontractors submit monthly hours worked and injury data by the 5th of the following month.

# Section 20. Stop Work Authority

20.1  Every person on site has the authority and the obligation to stop work that is unsafe. No one will be disciplined for a good-faith stop.

20.2  Work stopped for an imminent danger restarts only after the superintendent and the subcontractor's foreman agree the hazard is controlled, and the Project Safety Manager is informed.

# Section 21. Drug and Alcohol

21.1  Subcontractors shall maintain a drug and alcohol program with pre-access, post-incident and reasonable-suspicion testing.

# Section 22. Performance Evaluation

22.1  Each subcontractor is evaluated at completion of its scope on safety performance, corrective action timeliness and plan quality. The result sets its status for future bids: Preferred, Approved or Conditional.

22.2  A serious injury or a serious near miss places the subcontractor on Conditional status with a performance improvement plan for 12 months.

Appendix A (forms) is maintained separately in the Brightwater forms library and is not reproduced here.